TRUCKING CHECK OFF SHEET

# Read appropriate manuals on equipment you  will be operating, If you do not

understand the manual, ask your supervisor for operating instructions.

- Pre-trip inspection:

- Check water and oil levels when cold. Also check fan belt tightness and oil

and antifreeze leaks.

2.	Check lights, tires, and wheel lug nuts for any that are broken or loose.

- Check body and glass or mirrors for break or cracks.

- Fill out pre-trip logs or company forms.

- Operation:

1.	Depress clutch pedal.

- Check manual, kill throttle where applicable start engine. If engine doesn’t

start in 30 seconds, release starter button or key. Allow started truck to cool

for 2-3 minutes before trying to start engine again.

- Watch gauge proper oil pressure. If you feel that pressure is not acceptable for

type of engine, shut down engine immediately and check for any oil leaks.

- Check air brake pedal and listen for air leaks while watching air pressure gauge

to see if it has at least 115 lbs. Pressure and doesn’t drop more than 20 lbs. in a

one minute span.

- Allow adequate warm-up time before operation.

- Release air brake valve on control panel in truck and look in mirrors on both

sides to make sure brakes are released and tires are not dragging.

7.	Never operate unit beyond recommended limits.

8.	Do not slow unauthorized persons to operate or drive in company vehicles.

9.	Absolutely no personnel ride on outside of moving vehicle.

- Always be aware of potential hazards and drive defensively.

- Look vehicle and do post-trip inspection and fill out forms and turn into supervisory personnel.

D	Safety:

- When working on vehicle, turn off engine set air brakes and put transmission in

gear to immobilize vehicle.

- When checking under dump boxes or any  thing overhead, place box brakes or

wood blocking under this equipment to keep from falling on individuals.

(Do not leave dump boxes in air overnight without blocking.)

- If vehicle is unsafe, do not operate. Contact supervisory personnel and they

will determine if unit needs to be repaired on RED TAG for out-of-operation.

OTHER_________________________________________________________________

SUPERVISORS SIGNATURE___________________________DATE______________

EMPLOYEES SIGNATURE_____________________________DATE______________